EXP 10

clc;

clear; close all;

% Performance Analysis of FM System under Gaussian White Noise

%% 1. System Parameters

Fs = 200000;		% Sampling frequency (Hz) T = 0.1;	% Simulation time (seconds)

t = 0:1/Fs:T-1/Fs;

fm = 200;		% Message frequency (Hz) fc = 10000;			% Carrier frequency (Hz) Am = 1;	% Message amplitude

Ac = 1;	% Carrier amplitude

kf = 2000;	% Frequency sensitivity (Hz/V)

SNR_dB = [0 5 10 15 20]; % Input SNR values

%% 2. Generate Message Signal m = Am*cos(2*pi*fm*t);

%% 3. Generate FM Signal

phase = 2*pi*fc*t + 2*pi*kf*cumtrapz(t,m); sFM = Ac*cos(phase);

%% 4. Initialize Performance Variables MSE = zeros(size(SNR_dB));

OutputSNR = zeros(size(SNR_dB));

% Low-pass filter for recovered message [b,a] = butter(5, 1000/(Fs/2), 'low');

%% 5. Plot Original Message and FM Signal figure('Name','FM Signal Generation');

subplot(2,1,1);

plot(t,m,'b');

title('Original Message Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(2,1,2);

plot(t,sFM,'r');

title('Frequency Modulated Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

%% 6. Noisy Channel and FM Demodulation figure('Name','FM Performance Under AWGN');

for k = 1:length(SNR_dB)

% Add Gaussian white noise

signalPower = mean(sFM.^2);

noisePower = signalPower / 10^(SNR_dB(k)/10); noise = sqrt(noisePower)*randn(size(sFM));

received = sFM + noise;

% FM demodulation using phase differentiation analyticSignal = hilbert(received);

unwrappedPhase = unwrap(angle(analyticSignal));

instFreq = [diff(unwrappedPhase)*Fs/(2*pi), 0];

% Remove carrier frequency detected = instFreq - fc;

% Low-pass filtering

detected = filtfilt(b,a,detected);

% Remove numerical offset

detected = detected - mean(detected);

% Normalize to original message amplitude detected = detected * ...

(Am / (max(abs(detected)) + eps));

% Ignore edge samples during performance calculation idx = round(0.01*Fs):round(0.09*Fs);

errorSignal = m(idx) - detected(idx);

% Calculate MSE and output SNR MSE(k) = mean(errorSignal.^2); OutputSNR(k) = 10*log10( ...

mean(m(idx).^2)/(MSE(k) + eps));

% Plot a short section of detected signal subplot(length(SNR_dB),1,k); plot(t(idx),m(idx),'b');

hold on;

plot(t(idx),detected(idx),'r');

title(sprintf('FM Detection at Input SNR = %d dB', ... SNR_dB(k)));

xlabel('Time (s)'); ylabel('Amplitude');

legend('Original','Detected'); grid on;

end

%% 7. Plot Performance Graphs

figure('Name','FM Performance Evaluation');

subplot(2,1,1);

semilogy(SNR_dB,MSE,'bo-','LineWidth',1.5); title('MSE versus Input SNR');

xlabel('Input SNR (dB)'); ylabel('Mean Squared Error'); grid on;

subplot(2,1,2);

plot(SNR_dB,OutputSNR,'rs-','LineWidth',1.5); title('Output SNR versus Input SNR'); xlabel('Input SNR (dB)');

ylabel('Output SNR (dB)'); grid on;

%% 8. Display Results

fprintf('\nFM Performance Analysis Results\n'); fprintf('	\n');

fprintf('Input SNR (dB)  MSE	Output SNR (dB)\n');

fprintf('	\n');

for k = 1:length(SNR_dB)

fprintf('%10.1f  %8.5f  %10.2f\n', ...

SNR_dB(k), MSE(k), OutputSNR(k));

end

disp('FM performance analysis completed successfully.');