EXP 16

clc;

clear; close all;

% Simulation and Analysis of Uniform and Non-Uniform Quantization

%% 1. System Parameters

Fs = 10000;	% Sampling frequency t = 0:1/Fs:0.05;		% Time vector

fm = 100;	% Message frequency (Hz)

A = 1;	% Signal amplitude

nBits = 3;	% Number of quantization bits

L = 2^nBits;	% Number of quantization levels

%% 2. Generate Message Signal x = A*sin(2*pi*fm*t);

%% 3. Uniform Quantization xmin = -A;

xmax = A;

delta = (xmax-xmin)/L;

% Mid-rise uniform quantizer indexU = floor((x-xmin)/delta); indexU = min(max(indexU,0),L-1);

xU = xmin + (indexU+0.5)*delta;

%% 4. Non-Uniform Quantization

% Mu-law compression, uniform quantization, expansion mu = 255;

% Compress the normalized input signal

xComp = sign(x).*log(1+mu*abs(x)/A)/log(1+mu);

% Uniformly quantize the compressed signal deltaC = 2/L;

indexN = floor((xComp+1)/deltaC); indexN = min(max(indexN,0),L-1);

xCompQ = -1 + (indexN+0.5)*deltaC;

% Expand the quantized signal xN = sign(xCompQ).*A.* ...

((1+mu).^abs(xCompQ)-1)/mu;

%% 5. Calculate Quantization Errors errorU = x-xU;

errorN = x-xN;

%% 6. Calculate MSE

MSE_U = mean(errorU.^2); MSE_N = mean(errorN.^2);

%% 7. Calculate SQNR

signalPower = mean(x.^2);

SQNR_U = 10*log10(signalPower/MSE_U); SQNR_N = 10*log10(signalPower/MSE_N);

%% 8. Plot Original and Quantized Signals figure('Name','Quantization Analysis');

subplot(3,1,1);

plot(t,x,'b','LineWidth',1.3); title('Original Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(3,1,2);

stairs(t,xU,'r','LineWidth',1.1); hold on;

plot(t,x,'b--');

title('Uniform Quantization'); xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized','Original'); grid on;

subplot(3,1,3);

stairs(t,xN,'m','LineWidth',1.1); hold on;

plot(t,x,'b--');

title('Non-Uniform Quantization (Mu-law)'); xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized','Original'); grid on;

%% 9. Plot Quantization Errors

figure('Name','Quantization Error Analysis');

subplot(2,1,1);

plot(t,errorU,'r');

title('Uniform Quantization Error'); xlabel('Time (s)');

ylabel('Error'); grid on;

subplot(2,1,2);

plot(t,errorN,'m');

title('Non-Uniform Quantization Error'); xlabel('Time (s)');

ylabel('Error'); grid on;

%% 10. Display Results

fprintf('\nQuantization Analysis Results\n'); fprintf('	\n');

fprintf('Number of Bits	 = %d\n',nBits); fprintf('Quantization Levels = %d\n',L); fprintf('Uniform MSE	= %.6f\n',MSE_U); fprintf('Non-Uniform MSE			= %.6f\n',MSE_N); fprintf('Uniform SQNR		= %.2f dB\n',SQNR_U);

fprintf('Non-Uniform SQNR	= %.2f dB\n',SQNR_N);

disp('Uniform and non-uniform quantization completed.');