EXP 22

clc;

clear; close all;

%% 1. System Parameters

N = 10000;	% Number of bits

SNR_dB = 10;	% Signal-to-noise ratio in dB

%% 2. Generate Random Binary Data bits = randi([0 1], N, 1);

%% 3. BPSK Modulation

% Bit 0 -> -1

% Bit 1 -> +1

tx_symbols = 2*bits - 1;

%% 4. Add AWGN Noise

% For BPSK with unit bit energy, noise variance per

% real dimension is N0/2 = 1/(2*Eb/N0). EbN0 = 10^(SNR_dB/10);

noise = sqrt(1/(2*EbN0)) * randn(N, 1);

rx_symbols = tx_symbols + noise;

%% 5. Constellation Diagram Before Noise figure;

plot(real(tx_symbols), imag(tx_symbols), 'bo', ... 'MarkerSize', 5);

grid on; axis equal;

xlim([-1.5 1.5]);

ylim([-1.5 1.5]);

xlabel('In-Phase (I)'); ylabel('Quadrature (Q)');

title('BPSK Constellation Before Noise');

%% 6. Constellation Diagram After Noise figure;

plot(real(rx_symbols), imag(rx_symbols), 'r.', ... 'MarkerSize', 8);

grid on; axis equal; xlim([-2 2]);

ylim([-1.5 1.5]);

xlabel('In-Phase (I)'); ylabel('Quadrature (Q)');

title(['BPSK Constellation With AWGN, SNR = ', ... num2str(SNR_dB), ' dB']);

%% 7. BPSK Demodulation

detected_bits = rx_symbols > 0;

%% 8. Calculate Bit Error Rate

errors = sum(bits ~= detected_bits); BER_simulated = errors/N;

% Theoretical BER for coherent BPSK in AWGN BER_theoretical = 0.5 * erfc(sqrt(EbN0));

fprintf('BPSK Modulation Results\n'); fprintf('	\n');

fprintf('Number of bits		= %d\n', N); fprintf('SNR (Eb/N0)	= %d dB\n', SNR_dB); fprintf('Number of bit errors = %d\n', errors);

fprintf('Simulated BER	= %.6f\n', BER_simulated); fprintf('Theoretical BER	 = %.6e\n', BER_theoretical);

%% 9. BER Performance for Different SNR Values SNR_range = 0:2:12;

BER_sim = zeros(size(SNR_range)); BER_theory = zeros(size(SNR_range));

for k = 1:length(SNR_range) ebn0 = 10^(SNR_range(k)/10);

% Generate independent noise for each SNR n = sqrt(1/(2*ebn0)) * randn(N,1);

received = tx_symbols + n;

detected = received > 0;

BER_sim(k) = sum(bits ~= detected)/N; BER_theory(k) = 0.5*erfc(sqrt(ebn0));

end

figure;

semilogy(SNR_range, max(BER_sim, 1/N), 'bo-', ... 'LineWidth', 1.5, 'MarkerSize', 6);

hold on;

semilogy(SNR_range, BER_theory, 'r*-', ... 'LineWidth', 1.5, 'MarkerSize', 6);

grid on; xlabel('E_b/N_0 (dB)');

ylabel('Bit Error Rate (BER)');

title('BPSK BER Performance in AWGN');

legend('Simulated BER', 'Theoretical BER', ... 'Location', 'southwest');