EXP 24

clc;

clear; close all;

%% 1. System Parameters

Nbits = 100000;	% Number of transmitted bits SNR_dB = 0:2:20;		% SNR range

fc = 100e3;	% Carrier frequency (Hz)

Fs = 1e6;	% Sampling frequency (Hz)

Rs = 50e3;	% Symbol rate (symbols/sec) Ts = 1/Rs;

% QPSK requires two bits per symbol Nbits = 2*floor(Nbits/2);

rng(1);

%% 2. Generate Random Binary Data bits = randi([0 1], Nbits, 1);

% Serial-to-parallel conversion bitPairs = reshape(bits, 2, []).';

% QPSK mapping with unit average symbol power txSymbols = ((1 - 2*bitPairs(:,1)) + ...

1j*(1 - 2*bitPairs(:,2))) / sqrt(2);

Nsym = length(txSymbols);

%% 3. Passband Modulation

% Each symbol is represented by one sample in this

% simplified passband-equivalent simulation. t = (0:Nsym-1)' * Ts;

txPassband = real(txSymbols .* exp(1j*2*pi*fc*t));

figure;

plot(t(1:200)*1e3, txPassband(1:200)); xlabel('Time (ms)');

ylabel('Amplitude');

title('Transmitted QPSK Passband Signal'); grid on;

%% 4. Multipath Rayleigh Fading Channel

% Three-tap discrete-time channel with delayed paths h = [0.9, 0.45, 0.2] .* ...

exp(1j*[0, 0.7, -1.1]);

h = h / norm(h);

% Complex baseband equivalent multipath channel rxChannel = filter(h, 1, txSymbols);

%% 5. BER Without Equalization BER_noEQ = zeros(size(SNR_dB)); BER_EQ = zeros(size(SNR_dB));

% Zero-forcing equalizer for the three-tap channel

% For each symbol, use a linear MMSE equalizer

% with a fixed channel model. L = 11;

H = convmtx(h(:), L);

H = H(1:L+length(h)-1, :);

% Select the center symbol response as the target delay = 2;

d = zeros(size(H,1), 1); d(delay+1) = 1;

%% 6. Add AWGN, Equalize and Calculate BER for k = 1:length(SNR_dB)

snrLinear = 10^(SNR_dB(k)/10);

% Noise variance based on average received signal power noisePower = mean(abs(rxChannel).^2) / snrLinear;

noise = sqrt(noisePower/2) * ...

(randn(size(rxChannel)) + 1j*randn(size(rxChannel)));

rx = rxChannel + noise;

%% A. Detection Without Equalization

% Simple one-tap channel compensation using main path rxNoEQ = rx / h(1);

detectedNoEQ = zeros(Nsym, 2); detectedNoEQ(:,1) = real(rxNoEQ) < 0;

detectedNoEQ(:,2) = imag(rxNoEQ) < 0;

bitsNoEQ = reshape(detectedNoEQ.', [], 1); BER_noEQ(k) = mean(bitsNoEQ ~= bits);

%% B. MMSE Equalization

% Construct a linear MMSE equalizer

% Using the channel impulse response p = 2;

Hf = zeros(L, L);

for row = 1:L for col = 1:L

idx = row - col + 1;

if idx >= 1 && idx <= length(h) Hf(row,col) = h(idx);

end end

end

% Regularized inverse for noise robustness w = (Hf' * Hf + noisePower * eye(L)) \ ...

(Hf' * eye(L));

% Equalize received signal using a simple linear filter rxEQ = filter(conj(flipud(w(:,p))), 1, rx);

% Compensate for equalizer delay approximately rxEQ = rxEQ(3:end);

txRef = txSymbols(1:length(rxEQ));

detectedEQ = zeros(length(rxEQ), 2); detectedEQ(:,1) = real(rxEQ) < 0; detectedEQ(:,2) = imag(rxEQ) < 0;

bitsEQ = reshape(detectedEQ.', [], 1);

bitsRef = reshape([real(txRef)<0, imag(txRef)<0].', [], 1);

nCompare = min(length(bitsEQ), length(bitsRef));

BER_EQ(k) = mean(bitsEQ(1:nCompare) ~= bitsRef(1:nCompare)); end

%% 7. Plot BER Performance figure;

semilogy(SNR_dB, max(BER_noEQ, 1/Nbits), 'ro-', ... 'LineWidth', 1.5);

hold on;

semilogy(SNR_dB, max(BER_EQ, 1/Nbits), 'bs-', ... 'LineWidth', 1.5);

grid on;

xlabel('SNR (dB)');

ylabel('Bit Error Rate (BER)');

title('QPSK BER under Multipath Fading');

legend('Without Equalization', 'With Equalization', ... 'Location', 'southwest');

%% 8. Display Results

fprintf('\nQPSK Multipath Fading Simulation Results\n'); fprintf('	\n');

for k = 1:length(SNR_dB)

fprintf('SNR = %2d dB | BER without EQ = %.6f | BER with EQ = %.6f\n', ... SNR_dB(k), BER_noEQ(k), BER_EQ(k));

end

fprintf('\nSimulation completed.\n');