EXP 25

lc;

clear; close all;

% Simulation of Convolutional Coding

% and Analysis of Encoded Output Bit Sequences

% Step 1: Parameters

N = 20;	% Number of input bits constraint_length = 3;

memory = constraint_length - 1;

% Generator polynomials: 111 and 101

g1 = [1 1 1];

g2 = [1 0 1];

% Step 2: Generate Random Input Bits data = randi([0 1], 1, N);

% Add tail bits to return encoder to zero state input_bits = [data zeros(1, memory)];

% Step 3: Convolutional Encoding state = zeros(1, memory); encoded = [];

for i = 1:length(input_bits)

reg = [input_bits(i) state];

out1 = mod(sum(reg .* g1), 2); out2 = mod(sum(reg .* g2), 2);

encoded = [encoded out1 out2];

state = reg(1:memory); end

% Step 4: Display Input and Encoded Bits disp('Original Input Bits:');

disp(data);

disp('Input Bits with Tail Bits:'); disp(input_bits);

disp('Convolutionally Encoded Output Bits:'); disp(encoded);

% Step 5: BPSK Modulation tx = 1 - 2*encoded;

% Step 6: Add AWGN SNR_dB = 4;

signal_power = mean(tx.^2);

noise_power = signal_power / (10^(SNR_dB/10));

rx = tx + sqrt(noise_power)*randn(size(tx));

% Step 7: Hard Decision Detection received_bits = rx < 0;

disp('Received Bits after Hard Decision:'); disp(received_bits);

% Step 8: Viterbi Decoding num_states = 2^memory;

num_steps = length(received_bits)/2;

path_metric = inf(num_states, num_steps+1); path_metric(1,1) = 0;

previous_state = zeros(num_states, num_steps); previous_input = zeros(num_states, num_steps);

for t = 1:num_steps

r = received_bits(2*t-1:2*t);

for s = 0:num_states-1

if isinf(path_metric(s+1,t)) continue;

end

% Convert state to two memory bits state_bits = [bitget(s,2), bitget(s,1)];

for u = 0:1

reg = [u state_bits];

o1 = mod(sum(reg .* g1), 2);

o2 = mod(sum(reg .* g2), 2); out = [o1 o2];

% Next state

next_bits = reg(1:memory);

ns = next_bits(1)*2 + next_bits(2);

% Hamming distance distance = sum(out ~= r);

new_metric = path_metric(s+1,t) + distance;

if new_metric < path_metric(ns+1,t+1) path_metric(ns+1,t+1) = new_metric; previous_state(ns+1,t) = s; previous_input(ns+1,t) = u;

end end

end end

% Traceback from zero state because tail bits were added decoded = zeros(1, num_steps);

current_state = 0;

for t = num_steps:-1:1

decoded(t) = previous_input(current_state+1,t); current_state = previous_state(current_state+1,t);

end

decoded_data = decoded(1:N);

disp('Decoded Output Bits:'); disp(decoded_data);

% Step 9: Calculate Bit Error Rate

errors = sum(data ~= decoded_data);

BER = errors/N;

fprintf('\nNumber of Bit Errors = %d\n', errors); fprintf('Bit Error Rate (BER) = %.4f\n', BER);

% Step 10: Plot Input and Encoded Sequences figure;

subplot(3,1,1);

stairs(data, 'LineWidth', 1.5); grid on;

ylim([-0.2 1.2]);

title('Original Input Bit Sequence'); xlabel('Bit Index');

ylabel('Bit');

subplot(3,1,2);

stairs(encoded, 'LineWidth', 1.5); grid on;

ylim([-0.2 1.2]);

title('Convolutionally Encoded Output Sequence'); xlabel('Encoded Bit Index');

ylabel('Bit');

subplot(3,1,3);

stairs(decoded_data, 'LineWidth', 1.5);

grid on;

ylim([-0.2 1.2]);

title('Decoded Output Bit Sequence'); xlabel('Bit Index');

ylabel('Bit');