EXP 26

clc;

clear; close all;

%% 1. Define (7,4) Hamming Code

% Generator matrix G = [1 0 0 0 1 1 0;

0 1 0 0 1 0 1;

0 0 1 0 0 1 1;

0 0 0 1 1 1 1];

% Parity-check matrix H = [1 1 0 1 1 0 0;

1 0 1 1 0 1 0;

0 1 1 1 0 0 1];

%% 2. Generate Data Bits data = [1 0 1 1];

disp('Original Data Bits:'); disp(data);

%% 3. Encoding

% Codeword = data * G modulo 2 codeword = mod(data * G, 2);

disp('Encoded Codeword:'); disp(codeword);

%% 4. Introduce a Single-Bit Error received = codeword;

errorPosition = 3; % Position of the bit error received(errorPosition) = ...

mod(received(errorPosition) + 1, 2);

disp('Received Codeword with Error:'); disp(received);

%% 5. Calculate Syndrome Vector

% Syndrome = received * H transpose modulo 2 syndrome = mod(received * H', 2);

disp('Syndrome Vector:'); disp(syndrome);

%% 6. Detect Error Position if all(syndrome == 0)

disp('No error detected.'); corrected = received;

else

% Find the column of H matching the syndrome errorFound = false;

for i = 1:size(H, 2)

if isequal(syndrome', H(:, i)) errorPositionDetected = i; errorFound = true;

break; end

end

if errorFound

fprintf('Error detected at bit position %d\n', ... errorPositionDetected);

%% 7. Correct the Error corrected = received;

corrected(errorPositionDetected) = ...

mod(corrected(errorPositionDetected) + 1, 2);

else

disp('Error pattern is not a single-bit error.'); corrected = received;

end end

%% 8. Display Corrected Codeword disp('Corrected Codeword:');

disp(corrected);

%% 9. Extract Original Data Bits decodedData = corrected(1:4);

disp('Recovered Data Bits:'); disp(decodedData);

%% 10. Verify Correction

if isequal(corrected, codeword) disp('SUCCESS: Single-bit error corrected.');

else

disp('Correction unsuccessful.');

end

%% 11. Plot Bit Sequences

figure('Name', 'Hamming Code Error Correction');

subplot(4,1,1);

stem(data, 'filled');

title('Original Data Bits'); xlabel('Bit Position'); ylabel('Bit Value');

ylim([-0.2 1.2]);

grid on;

subplot(4,1,2);

stem(codeword, 'filled'); title('Encoded 7-Bit Codeword'); xlabel('Bit Position');

ylabel('Bit Value'); ylim([-0.2 1.2]);

grid on;

subplot(4,1,3);

stem(received, 'filled');

title('Received Codeword with Single-Bit Error'); xlabel('Bit Position');

ylabel('Bit Value'); ylim([-0.2 1.2]);

grid on;

subplot(4,1,4);

stem(corrected, 'filled');

title('Corrected Codeword'); xlabel('Bit Position');

ylabel('Bit Value'); ylim([-0.2 1.2]);

grid on;