EXP 30

clc;

clear; close all;

% Cellular Frequency Planning and Handover Performance

% under User Mobility Conditions

% Step 1: Define system parameters numCells = 7;

cellRadius = 500;	 % Cell radius in meters reuseFactor = 3;	% Frequency reuse factor numChannels = 60;		% Total available channels

handoverThreshold = -85;	% Handover threshold in dBm hysteresis = 5;	% Handover hysteresis in dB

% Step 2: Create cell locations angles = (0:5)*pi/3;

cellX = [0, 2*cellRadius*cos(angles)]; cellY = [0, 2*cellRadius*sin(angles)];

% Frequency group assignment

frequencyGroups = mod(0:numCells-1,reuseFactor) + 1;

% Step 3: Simulate mobile user movement numSteps = 300;

userX = linspace(-900,900,numSteps);

userY = 100*sin(linspace(0,2*pi,numSteps));

% Path-loss model parameters

Pt = 30;	% Transmit power in dBm

PL0 = 40;	% Reference path loss in dB

d0 = 1;	% Reference distance in meters pathLossExponent = 3.5;

servingCell = zeros(1,numSteps);

signalStrength = zeros(numCells,numSteps); handoverCount = 0;

successfulHandovers = 0;

failedHandovers = 0;

currentCell = 1;

% Step 4: Calculate received signal strength for t = 1:numSteps

for c = 1:numCells

distance = sqrt((userX(t)-cellX(c))^2 + ... (userY(t)-cellY(c))^2);

distance = max(distance,d0);

pathLoss = PL0 + 10*pathLossExponent*log10(distance/d0);

% Log-normal shadow fading shadowFading = 4*randn;

signalStrength(c,t) = Pt - pathLoss + shadowFading; end

% Find strongest cell

[bestRSSI,bestCell] = max(signalStrength(:,t));

% Handover decision with hysteresis if currentCell ~= bestCell

currentRSSI = signalStrength(currentCell,t);

if bestRSSI > currentRSSI + hysteresis && ... currentRSSI < handoverThreshold handoverCount = handoverCount + 1;

% Handover admission check (simplified) if rand > 0.05

currentCell = bestCell;

successfulHandovers = successfulHandovers + 1; else

failedHandovers = failedHandovers + 1; end

end end

servingCell(t) = currentCell; end

% Step 5: Calculate performance metrics

handoverSuccessRate = 100 * successfulHandovers / ... max(handoverCount,1);

handoverFailureRate = 100 * failedHandovers / ... max(handoverCount,1);

% Step 6: Display results

fprintf('\nCELLULAR FREQUENCY PLANNING\n');

fprintf('Number of cells = %d\n',numCells); fprintf('Cell radius = %d meters\n',cellRadius);

fprintf('Frequency reuse factor = %d\n',reuseFactor); fprintf('Total available channels = %d\n',numChannels); fprintf('Channels per reuse group (approx.) = %d\n', ...

floor(numChannels/reuseFactor));

fprintf('\nHANDOVER PERFORMANCE\n');

fprintf('Total handover attempts = %d\n',handoverCount); fprintf('Successful handovers = %d\n',successfulHandovers); fprintf('Failed handovers = %d\n',failedHandovers); fprintf('Handover success rate = %.2f%%\n',handoverSuccessRate); fprintf('Handover failure rate = %.2f%%\n',handoverFailureRate);

% Step 7: Plot cellular layout and mobile path figure;

subplot(2,2,1);

plot(cellX,cellY,'bo','MarkerFaceColor','b'); hold on;

plot(userX,userY,'r-','LineWidth',1.5); for c = 1:numCells

text(cellX(c),cellY(c),sprintf(' C%d (F%d)', ... c,frequencyGroups(c)));

end

title('Cellular Layout and Mobile User Path'); xlabel('X Position (m)');

ylabel('Y Position (m)');

legend('Base Stations','User Path'); grid on;

axis equal;

% Step 8: Plot received signal strength subplot(2,2,2);

plot(1:numSteps,signalStrength','LineWidth',1); hold on;

yline(handoverThreshold,'k--','Handover Threshold'); title('Received Signal Strength');

xlabel('Time Step'); ylabel('RSSI (dBm)'); grid on;

% Step 9: Plot serving cell changes subplot(2,2,3);

stairs(1:numSteps,servingCell,'LineWidth',1.5); title('Serving Cell During User Mobility');

xlabel('Time Step'); ylabel('Cell Number'); yticks(1:numCells); grid on;

% Step 10: Plot handover performance subplot(2,2,4);

bar([successfulHandovers failedHandovers]); set(gca,'XTickLabel',{'Handover Results'});

legend('Successful','Failed'); title('Handover Performance'); ylabel('Number of Handovers'); grid on;