EXP 7

clc;

clear; close all;

% Calculation and Verification of Frequency Deviation

% and Modulation Index of FM Signals Using MATLAB

%% 1. System Parameters

Fs = 200000;	% Sampling frequency (Hz)

T = 0.05;	% Simulation duration (seconds) t = 0:1/Fs:T-1/Fs;		% Time vector

Am = 1;	% Message signal amplitude (V) fm = 200;		% Message frequency (Hz)

Ac = 1;	% Carrier amplitude (V)

fc = 5000;	% Carrier frequency (Hz)

kf = 1000;	% Frequency sensitivity (Hz/V)

%% 2. Generate Message Signal m = Am*cos(2*pi*fm*t);

%% 3. Calculate Frequency Deviation delta_f = kf*Am;

%% 4. Calculate FM Modulation Index beta = delta_f/fm;

%% 5. Generate FM Signal

% Instantaneous phase of FM signal

phase_FM = 2*pi*fc*t + 2*pi*kf*cumtrapz(t,m);

% FM modulated signal

sFM = Ac*cos(phase_FM);

%% 6. Calculate Instantaneous Frequency

% Numerical derivative of instantaneous phase inst_freq = [diff(phase_FM)*Fs/(2*pi), NaN];

%% 7. Verify Frequency Deviation f_max = fc + delta_f;

f_min = fc - delta_f;

fprintf('\n===== FM Parameter Calculation =====\n'); fprintf('Carrier Frequency  = %.2f Hz\n',fc);

fprintf('Message Frequency = %.2f Hz\n',fm); fprintf('Message Amplitude = %.2f V\n',Am); fprintf('Frequency Sensitivity = %.2f Hz/V\n',kf);

fprintf('Maximum Frequency Dev. = %.2f Hz\n',delta_f); fprintf('FM Modulation Index	= %.2f\n',beta); fprintf('Maximum Instantaneous F = %.2f Hz\n',f_max); fprintf('Minimum Instantaneous F = %.2f Hz\n',f_min);

%% 8. Plot Message Signal

figure('Name','FM Frequency Deviation and Modulation Index');

subplot(4,1,1);

plot(t,m,'b','LineWidth',1.2); title('Message Signal'); xlabel('Time (s)');

ylabel('Amplitude (V)'); grid on;

%% 9. Plot FM Signal subplot(4,1,2);

plot(t,sFM,'r');

title('Frequency Modulated Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

%% 10. Plot Instantaneous Frequency subplot(4,1,3);

plot(t,inst_freq,'k','LineWidth',1.1); hold on;

yline(fc,'b--','Carrier Frequency');

yline(f_max,'r--','Maximum Frequency'); yline(f_min,'g--','Minimum Frequency');

title('Instantaneous Frequency of FM Signal'); xlabel('Time (s)');

ylabel('Frequency (Hz)'); grid on;

ylim([fc-1.5*delta_f fc+1.5*delta_f]);

%% 11. Frequency Spectrum N = length(sFM);

f = (-floor(N/2):ceil(N/2)-1)*(Fs/N); S = abs(fftshift(fft(sFM))/N);

subplot(4,1,4);

plot(f/1000,S,'m');

xlim([0 12]);

title('FM Frequency Spectrum');

xlabel('Frequency (kHz)'); ylabel('Magnitude');

grid on;

%% 12. Verification

fprintf('\n===== Verification =====\n');

fprintf('Theoretical Frequency Deviation = %.2f Hz\n',delta_f); fprintf('Theoretical Modulation Index = %.2f\n',beta); fprintf('Expected Frequency Range	= %.2f to %.2f Hz\n', ...

f_min,f_max);

disp('FM simulation and verification completed successfully.');