EXP 9

clc;

clear; close all;

% Performance Evaluation of AM Generation and Detection

% under Noisy Channel Conditions Using MATLAB

%% 1. System Parameters

Fs = 200000;	% Sampling frequency (Hz)

T = 0.05;	% Simulation duration (seconds) t = 0:1/Fs:T-1/Fs;

fm = 200;	% Message frequency (Hz)

fc = 10000;	% Carrier frequency (Hz)

Am = 1;	% Message amplitude

Ac = 1;	% Carrier amplitude

mu = 0.7;	% AM modulation index

SNR_dB = [0 5 10 15 20];	% SNR values in dB

%% 2. Generate Message Signal m = Am*cos(2*pi*fm*t);

%% 3. Generate AM Signal

sAM = Ac*(1 + mu*m/Am).*cos(2*pi*fc*t);

%% 4. Plot Original Signals

figure('Name','AM Signal Generation');

subplot(2,1,1);

plot(t,m,'b','LineWidth',1.2);

title('Original Message Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(2,1,2);

plot(t,sAM,'r');

title('AM Modulated Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

%% 5. Initialize Performance Variables MSE = zeros(size(SNR_dB));

OutputSNR = zeros(size(SNR_dB));

% Low-pass filter for envelope smoothing [b,a] = butter(5, 1500/(Fs/2), 'low');

%% 6. Noisy Channel and AM Detection figure('Name','AM Detection Under Noise');

for k = 1:length(SNR_dB)

% Add AWGN

signalPower = mean(sAM.^2);

noisePower = signalPower / (10^(SNR_dB(k)/10)); noise = sqrt(noisePower)*randn(size(sAM));

received = sAM + noise;

% Envelope detection using Hilbert transform envelope = abs(hilbert(received));

% Remove DC component

detected = envelope - mean(envelope);

% Low-pass filtering

detected = filtfilt(b,a,detected);

% Normalize to message amplitude detected = detected / mu;

% Remove small numerical offset detected = detected - mean(detected);

% Calculate MSE

errorSignal = m - detected;

MSE(k) = mean(errorSignal.^2);

% Estimate output SNR OutputSNR(k) = 10*log10( ...

mean(m.^2)/(mean(errorSignal.^2) + eps));

% Plot received and detected signals subplot(length(SNR_dB),1,k);

plot(t,m,'b','LineWidth',1.1); hold on;

plot(t,detected,'r');

title(sprintf('AM Detection at Input SNR = %d dB', ... SNR_dB(k)));

xlabel('Time (s)');

ylabel('Amplitude');

legend('Original','Detected'); grid on;

end

%% 7. Performance Evaluation

figure('Name','AM System Performance');

subplot(2,1,1);

plot(SNR_dB,MSE,'bo-','LineWidth',1.5); title('Mean Squared Error vs Input SNR'); xlabel('Input SNR (dB)');

ylabel('MSE'); grid on;

subplot(2,1,2);

plot(SNR_dB,OutputSNR,'rs-','LineWidth',1.5); title('Output SNR vs Input SNR');

xlabel('Input SNR (dB)'); ylabel('Output SNR (dB)'); grid on;

%% 8. Display Results

fprintf('\nAM Performance Evaluation Results\n');

| fprintf(' |  | \n'); |
| --- | --- | --- |
| fprintf('Input SNR (dB) | MSE | Output SNR (dB)\n'); |
| fprintf(' |  | \n'); |

for k = 1:length(SNR_dB)

fprintf('%10.1f  %8.5f  %10.2f\n', ...

SNR_dB(k), MSE(k), OutputSNR(k));

end

disp('AM generation and noisy-channel evaluation completed.');