EXP 4

clc;

clear; close all;

% MATLAB Simulation of Superheterodyne Receiver for AM

% Communication System with Frequency Mixing and Filtering

%% 1. System Parameters

Fs = 500000;	% Sampling frequency (Hz) t = 0:1/Fs:0.02;	 % Time vector

fm = 1000;	% Message frequency (Hz) fc = 100000;		% RF carrier frequency (Hz)

fLO = 110000;	% Local oscillator frequency (Hz) fIF = abs(fLO - fc); % Intermediate frequency (Hz)

Am = 1;	% Message amplitude

Ac = 1;	% Carrier amplitude

mu = 0.7;	% Modulation index

%% 2. Generate Message Signal m = Am*cos(2*pi*fm*t);

%% 3. Generate AM Signal

sAM = Ac*(1 + mu*m/Am).*cos(2*pi*fc*t);

%% 4. Frequency Mixing

% Mixer produces sum and difference frequencies localOscillator = cos(2*pi*fLO*t);

mixerOutput = 2*sAM.*localOscillator;

%% 5. Intermediate Frequency (IF) Filtering

% IF = 10 kHz

% Select frequencies around the IF carrier

N = length(t);

f = (-floor(N/2):ceil(N/2)-1)*(Fs/N);

MIX_F = fftshift(fft(mixerOutput));

% Ideal bandpass filter centered at +/- IF IF_BW = 6000;

HIF = double(abs(abs(f)-fIF) <= IF_BW/2);

IF_F = MIX_F.*HIF;

IF_signal = real(ifft(ifftshift(IF_F)));

%% 6. AM Demodulation Using Envelope Detection

% Hilbert transform gives the signal envelope analyticSignal = hilbert(IF_signal);

envelope = abs(analyticSignal);

% Remove DC component

recovered = envelope - mean(envelope);

% Normalize recovered signal

recovered = recovered/max(abs(recovered));

%% 7. Plot Time-Domain Signals

figure('Name','Superheterodyne AM Receiver');

subplot(5,1,1);

plot(t,m,'b');

title('Original Message Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(5,1,2);

plot(t,sAM,'r');

title('AM Modulated RF Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(5,1,3);

plot(t,mixerOutput,'k'); title('Mixer Output'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(5,1,4);

plot(t,IF_signal,'m');

title('Intermediate Frequency (IF) Signal'); xlabel('Time (s)');

ylabel('Amplitude'); grid on;

subplot(5,1,5);

plot(t,recovered,'g','LineWidth',1.2); title('Recovered Message Signal');

xlabel('Time (s)'); ylabel('Amplitude'); grid on;

%% 8. Frequency Spectrum figure('Name','Frequency Spectrum');

subplot(3,1,1); plot(f/1000,abs(fftshift(fft(sAM)))/N); xlim([0 150]);

title('AM Signal Spectrum'); xlabel('Frequency (kHz)'); ylabel('Magnitude');

grid on;

subplot(3,1,2); plot(f/1000,abs(MIX_F)/N); xlim([0 250]);

title('Mixer Output Spectrum'); xlabel('Frequency (kHz)'); ylabel('Magnitude');

grid on;

subplot(3,1,3); plot(f/1000,abs(IF_F)/N); xlim([0 30]);

title('IF Filter Output Spectrum'); xlabel('Frequency (kHz)'); ylabel('Magnitude');

grid on;

%% 9. Display Parameters

fprintf('RF Carrier Frequency = %.2f kHz\n',fc/1000); fprintf('Local Oscillator Frequency = %.2f kHz\n',fLO/1000); fprintf('Intermediate Frequency = %.2f kHz\n',fIF/1000); fprintf('Message Frequency = %.2f kHz\n',fm/1000);

disp('Superheterodyne AM receiver simulation completed.');