EXP 28

clc;

clear; close all;

% DSSS Communication System

% PN Code Generation, Spreading and Signal Recovery

% Step 1: Generate input data data = [1 0 1 1 0 1 0 0];

N = length(data);

% Convert binary data to bipolar form data_bipolar = 2*data - 1;

% Step 2: Generate PN sequence using 3-bit LFSR reg = [1 1 1];

pn = zeros(1,7);

for k = 1:7

pn(k) = reg(3);

feedback = xor(reg(3),reg(1)); reg = [feedback reg(1:2)];

end

% Convert PN sequence to bipolar form pn_bipolar = 2*pn - 1;

L = length(pn);

% Step 3: Spread the data

spread_signal = zeros(1,N*L);

for k = 1:N

idx = (k-1)*L + (1:L);

spread_signal(idx) = data_bipolar(k)*pn_bipolar; end

% Step 4: Add AWGN noise SNR_dB = 10;

signal_power = mean(spread_signal.^2); noise_power = signal_power / (10^(SNR_dB/10));

noise = sqrt(noise_power)*randn(size(spread_signal));

received_signal = spread_signal + noise;

% Step 5: Despread and recover data recovered_data = zeros(1,N);

for k = 1:N

idx = (k-1)*L + (1:L);

received_chips = received_signal(idx);

correlation = sum(received_chips .* pn_bipolar);

if correlation > 0

recovered_data(k) = 1; else

recovered_data(k) = 0; end

end

% Step 6: Display results disp('Original Data:'); disp(data);

disp('Generated PN Sequence:'); disp(pn);

disp('Recovered Data:'); disp(recovered_data);

if isequal(data,recovered_data) disp('Data Recovery Successful');

else

disp('Some Bits Were Recovered Incorrectly'); end

% Step 7: Plot signals figure;

subplot(4,1,1);

stairs(0:N-1,data,'LineWidth',1.5); title('Original Binary Data');

xlabel('Bit Index'); ylabel('Bit');

ylim([-0.2 1.2]);

grid on;

subplot(4,1,2);

stairs(0:L-1,pn,'LineWidth',1.5); title('Generated PN Code'); xlabel('Chip Index');

ylabel('Chip'); ylim([-0.2 1.2]);

grid on;

subplot(4,1,3);

plot(received_signal,'LineWidth',1);

title('Received DSSS Signal with AWGN'); xlabel('Chip Index');

ylabel('Amplitude'); grid on;

subplot(4,1,4);

stairs(0:N-1,recovered_data,'LineWidth',1.5); title('Recovered Binary Data');

xlabel('Bit Index'); ylabel('Bit');

ylim([-0.2 1.2]);

grid on;